DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 33					MANUFACTURED PLANTERS

UNIVERSAL PLANTERS

Part 1	General

- Summary

  - Section Includes:

    - 30-gallon round planter.

    - 100-gallon round planter.

    - 190-gallon round planter.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of planters.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using litter and recycling receptacles fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: one year from date of invoice against defects in materials and workmanship.

Part 2	Products

- Manufacturer

  - Basis-of-design product: provide Universal Planters based on the product named:

    - Universal Planters by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Planters

    - Materials:

      - Container walls and internal structure: stainless steel.

      - Liners: black polyethylene with UL94HB fire-rating.

      - Concrete bases: 3.25" thick (2.5" visible) concrete.

    - Finishes

      - Stainless steel container walls (Choose one finish below):

        - Sandstone.

        - Diamond.

        - Satin.

        - Polyester powdercoat (Select one option below):

            - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

    - Body Patterns (Choose one option below):

      - No Pattern

      - Horizon Eco-Etch

      - Fade Perforation

    - Dimensions (Select model(s) below):

      - 30-gallon round planter: 23" diameter x 21.75" high.

      - 100-gallon round planter: 36" diameter x 28.5" high.1

      - 190-gallon round planter: 48" diameter x 34.5" high.1

NOTE: Unless otherwise indicated, heights shown are for freestanding versions with adjustable levelers. Optional concrete bases add an additional 1.75" to overall height.

NOTE 1: Dimensions shown for 100- and 190-gallon planters are for units resting directly on the substrate. Levelers and concrete bases are not available for these models.

    - Installation (Select one below):

      - Freestanding (provide levelers or concrete bases when freestanding is specified except for 100- and 190-gallon models).

      - Surface mount (mounting hardware kits are sold separately).

Part 3	Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION