Technical Bulletin

# Stack’d Series Battery Maintenance

Why Cycling Matters

Regular cycling helps balance individual battery cells and ensures accurate State of Charge (SOC) reporting. The HomeGrid Stack’d Series consists of 2 to 8 modules stacked together, each containing 15 battery cells in series. Cells can become imbalanced without periodic cycling, leading to inaccurate SOC calculations and reduced overall battery performance.

Optimal Usage Guidelines

- Keep the battery within an optimal charge range of 20%–100% SOC.

- Avoid deep discharges below 20% SOC, as this can accelerate cell degradation.

Recommended Maintenance Cycling

To maintain peak performance, HomeGrid recommends:

- At least one cycle per month, discharging from 100% to 90% SOC, then recharging to 100%.

- If the inverter supports a weekly cycle schedule, a weekly cycle is preferred for enhanced longevity.

Proper cycling helps maintain voltage stability across all cells, prolongs battery life, and ensures your HomeGrid system operates at full capacity.

For additional support, contact us at:

The HomeGrid Team

support@lithionbattery.com

1-855-753-3505

www.homegridenergy.com