NXP i.MX 8M Mini based

Board / HMI

Lea SBC /Agata HMI

User’s Manual

Elettronica GF s.r.l. Via Ca’ Bianca, 10 - 48018 – FAENZA (RA) Tel. (+39) 0546 622245 (r.a.) Fax (+39) 0546 622301

Web: www.elettronicagf.it   Mail: sales@elettronicagf.it

Disclaimer

No warranty of accuracy is given concerning the contents of the information contained in this product document.

Elettronica GF s.r.l. shall have no liability of any kind, express or implied, arising out of the use of the Information in this product document, including direct, indirect, special or consequential damages.

Elettronica GF reserves the right to change details in this document without notice

Elettronica GF s.r.l. reserves the right to make changes, without notice, to any product, including circuits and/or software described or contained in this document.

Elettronica GF s.r.l makes no commitment to update or upgrade the product or product documentation.

Elettronica GF s.r.l reserves the right to discontinue the product or product documentation at any time without notice.

All trademarks, service marks, logos, slogans, domain names and trade names (collectively "Marks") are the properties of their respective owners. Elettronica GF disclaims any proprietary interest in Marks other than its own

© Copyright 2024, Elettronica GF s.r.l. con socio unico - All Rights Reserved.

# Revision History

| Date | Doc. Rev. | Description |
| --- | --- | --- |
| 27-03-2025 | Rev.A01 | First Release |

# Table of contents

Revision History	3

Table of contents	4

1	Introduction	6

2	Reference Documents	7

3	Product identification	8

4	Acronyms and definitions	9

5	Board Block Diagram	10

6	Connections	11

6.1	Lea SBC	11

6.2	Agata HMI	11

6.3	Connectors, Jumpers and Switch List	12

7	System	13

7.1	Power Supply	13

7.1.1	Overview and Power Supply Block Diagram	13

7.1.2	Main Power Connector (CN22) – Molex MiniFit 2x2  87427 94V0	13

7.1.3	Power supply requirements	13

7.2	SOM	14

7.3	Reset and Boot Configuration	14

7.3.1	Power On Control	14

7.3.2	Reset Button (SW2)	14

7.3.3	Boot Mode Configuration Button (SW1)	14

8	I/O Interfaces	15

8.1	Debug USB Device FTDi (CN4) – micro USB Type B	15

8.2	10/100/1000 Ethernet Connectors (CN17, CN16)	15

8.3	microSD Card connector CN15 - GCT MEM2052-00-195-00-A	15

8.4	RTC	16

8.4.1	RTC Battery Socket (CN7)	16

8.5	USB	17

8.6	RS-232/RS-485/RS-422 UART Connector (CN18) – Molex 43045-0600	17

8.7	RS-485/RS-422 UART Connector (CN20) – Molex 43045-0600	19

8.8	HDMI Display Connector (CN21) - Molex 208658-1003	19

8.9	CAN-FD connector (CN19) – Molex 43045-0400	19

8.10	I/O Expansion connector (CN3) – Molex 87831-2020	20

8.10.1	I/O Expansion connector in default configuration	20

8.11	Audio Connector (CN8) – Molex MicroFit 43045-0400	22

8.12	Buzzer (BZ1)	22

8.13	M.2 Key E (CN11)	22

8.14	M.2 Key B (CN10)	22

8.15	Camera (CN2) – Hirose FH41-28S-0.5SH	23

8.16	nanoSIM Card Slot (CN23) – TE 2336582-1	24

9	Ordering Information	25

9.1	Lea SBC Ordering Codes	25

9.2	Agata HMI Ordering Codes	25

10	Legal notes	26

11	Contact Information	27

12	Annex A - Safety and Environmental Requirements	28

12.1	EMC	28

12.2	ESD	28

12.3	Environmental operating conditions	28

12.4	RoHS compliance	28

12.5	PCB Specification	28

12.6	Batteries	28

13	Annex B - Mechanical drawing	29

13.1	Lea SBC board mechanical drawing	29

13.2	Agata HMI board mechanical drawing	30

13.2.1	Agata HMI1xxxxxP	30

13.2.2	Agata HMI1xxxxxQ	31

13.2.3	Agata HMI1xxxxxR	32

13.2.4	Agata HMI5xxxxxP	33

13.2.5	Agata HMI5xxxxxQ	34

13.2.6	Agata HMi11xxxxxP	35

13.2.7	Agata HMi21xxxxxP	36

14	Annex C – Optical Characteristics	37

14.1	Display 7.1” (3HI4001)	37

14.2	Display 10.1” (3HI4005xxxxxP / 3HI4005xxxxxQ)	37

14.3	Display 10.1” (3HI4005xxxxxR)	38

14.4	Display 15.6” (3HI4011)	39

14.5	Display 21.5” (3HI4021)	40

# Introduction

This manual refers to the devices Lea SBC and Agata HMI which share the same board design based on NXP i.MX8M Plus Application Processor, based on Elettronica GF Eva SoM series:

- Lea SBC is a high-performance compact single board computer

- Agata HMI is a high-performance and flexible Panel PC solution, driven by the on-board Lea SBC

These boards may be equipped with the following interfaces and peripherals, depending on the chosen configuration:

| FEATURES - Scalable architectures customizable with all NXP i.MX 8M Plus variants - Available with different display 7”, 10.1”, 15.6”, 21.5” - NPU for AI applications available - Security subsystem with cryptographic accelerators - Customizable Panel PC - Fan less system - Boxed (CPT) and open frame version (RTP) - Panel Mount and Standard Vesa mount - Long longevity support - HTML 5 supported APPLICATIONS - Industrial Automation - Industrial remote controls - Construction machinery - Smart Retail - Intelligent Grid - Warehouse Systems - Vending machines - Robot Industries CPU - NXP i.MX8M Plus Dual/Quad+NPU up to 1,8 GHz MEMORY - Up to 4 GB LPDDR4 32 bit STORAGE / BOOT - Up to 64 GB eMMC - µSD slot DISPLAY AND TOUCH - Size available 7” - 10.1’’- 15,6” – 21,5’’ up to 1920x1080 - Resistive or capacitive touch screen option | PERIPHERALS1 - 2 x USB 3.0 Host connector - 1 x HDMI Display Connector - 2 x Ethernet 1 Gbps - 1 x CAN FD Interface with integrated Transceiver - 1 x RS-232/RS-422/RS-485 Sw selectable - 1 x RS-485 full duplex - 1 x M.2 key E Connector for Wi-Fi / BT Modules - 1 x M.2 key B Connector for LTE Modules - Buzzer - Temperature sensor - Onboard RTC - 1 x microSD card connector TEMPERATURE2 - Lea SBC - Industrial: From -40 °C to +85 °C - Commercial: From 0 °C to +70 °C - Agata HMI - Industrial: From -20 °C to +70 °C - Commercial: From 0°C to +50 °C SYSTEM Single +12-24 VDC ±10% power supply OPERATING SYSTEMS - Linux Embedded HMI DEVELOP SDK3 - HTML5 supported - QT |
| --- | --- |

________________________________________________________________________________

1 Please note that not all the functions may be available simultaneously, due to pin mux limitation of the CPU. We can check if our board suits your application, contact us for more information

2 These temperatures are the ambient operating temperature ranges for the components used into the board, with exception of i.MX8M series CPU which is specified on junction operating temperature range. Panel PC ambient operating temperature depends on the application and on the cooling measures applied

3 QT support is only under requests from Costumer, all responsibility and license costs are in charge on costumer

# Reference Documents

Following table lists referred documents in this user’s manual:

| N | Document | Link |
| --- | --- | --- |
| 1 | i.MX 8MP Reference Manual | Link |
| 2 | i.MX 8MP Datasheet | Link |
| 3 | AN12410: i.MX 8MP Power Consumption Application Note | Link |
| 4 | Elettronica_GF_SOM-3SM1009-DS_ENG_v.A02 |  |

Table 1

# Product identification

Each product has its own label containing the necessary information to correctly identify your product.

Figure 1

Description:

- Product model sale code

- Power supply electrical specification (input voltage range and maximum input current)

- Serial number

- MAC address 1

- MAC address 2

- QR-Code with the following format: S.N.; MAC1; MODEL

- Production date

- Company logo

- Disposal information

- Company name and address

# Acronyms and definitions

The following is a list of commonly occurring acronyms and abbreviations used throughout this document.

| Term | Definition |
| --- | --- |
| CAN | Controller Area Network |
| CMOS | Complementary Metal-Oxide Semiconductor |
| CPU | Central Processing Unit |
| DDR4 | Double Data Rate 4 RAM Memory |
| ECSPI | Enhanced Configurable Serial Peripheral Interface |
| EMI | ElectroMagnetic Interference |
| ESAI | Enhanced serial audio interface |
| ESDHC | Enhanced secure digital host controller |
| FEC | Fast Ethernet controller |
| GPIO | General Purpose Input Output |
| I2C | Inter-IC bus |
| IO | Input Output |
| JTAG | Joint Test Action Group |
| LCD | Liquid Crystal Display |
| LED | Light Emitting Diode |
| LSB | Least Significant Bit |
| MB | Mega Byte |
| MSB | Most Significant Bit |
| N/A | Not Applicable |
| NC | No connection |
| OS | Operating System |
| OTG | On-The-Go |
| PCB | Printed Circuit Board |
| PHY | Physical Transceiver |
| PLL | Phase-Locked Loop |
| PMIC | Power Management Integrated Circuit |
| PWM | Pulse Width Modulation |
| RTC | Real Time Clock |
| RX | Receive |
| SATA | Serial Advanced Technology Attachment |
| SDRAM | Synchronous Dynamic Random Access Memory |
| SBC | Single Board Computer |
| SOC | System On a Chip |
| SOM | System On Module |
| SPDIF | Sony Phillips Digital Interface |
| SPI | Serial Peripheral Interface |
| TX | Transmit |
| TTL | Transistor Transistor Logic |
| UART | Universal Asynchronous Receiver/Transmitter |
| USB | Universal Serial Bus |
| UTMI | USB Transceiver Macrocell Interface |

Table 2

# Board Block Diagram

Figure 2: 3SB4000 and 3HI40xx Board block diagram

# Connections

J1

J2

J4

J3

MEC8

MEC7

MEC6

MEC5

CN15

MEC2

MEC1

MEC3

MEC4

CN22

CN17

CN16

CN14

CN13

CN15

CN21

CN20

CN19

CN18

CN10

CN11

SW2

SW1

CN9

CN8

CN6

CN5

CN4

CN3

CN2

CN1

CN7

TP1

Lea SBC

Figure 3: 3SB4000 Layout – Top Side

## Agata HMI

The 3HI40xx Panel PC solution contains a 3SB4000 board with only a few connectors exposed, shown in the figure below.

CN22

CN17

CN16

CN15

CN14

CN13

CN21

CN20

CN18

CN19

Figure 4: 3HI40xx Layout – Bottom Side

## Connectors, Jumpers and Switch List

| Connector; Description. CN8; Audio Connector. CN7; RTC Battery Socket. CN5; MIPI-DSI Connector. CN4; Debug UART Connector. CN19; CAN-FD Connector. CN17; 10/100/1000 Ethernet Connector 1. CN16; 10/100/1000 Ethernet Connector 2. CN14; USB 3.0 Host Connector 1 (USB1). CN13; USB 3.0 Host Connector 2 (USB3). CN1; USB 2.0 Host Connector 3 (USB5). CN15; Micro SD Card Connector. CN2; MIPI CSI-2 Connector. CN18; MultiStd RS-232/ RS-422/RS-485 UART Connector. CN20; RS-422/RS-485 UART Connector. CN22; Main Power Connector. CN11; M.2 Key E Connector for Wi-Fi / BT modules. CN10; M.2 Key B Connector for LTE modules. CN3; Expansion Connector. CN21; HDMI Display Connector. CN23; Nano SIM Card Slot. | Connector; Description. J3,J4; RS-485 on CN18 120R Termination Enable Jumpers. J1; RS-485 on CN20 120R Termination Enable Jumper. SW1; Recovery Button. SW2; Reset Button. |
| --- | --- |

Table 3

# System

## Power Supply

### Overview and Power Supply Block Diagram

Agata HMI/ Lea SBC boards are powered by a single 12 ÷ 24 Vdc, ± 10% power supply.

The following image is the board power supply block diagram:

Figure 5

### Main Power Connector (CN22) – Molex MiniFit 2x2  87427 94V0

The following table shows the Main Power Connector pinout:

| Pin Number | Pin Name | I/O Type | Operating Voltage |
| --- | --- | --- | --- |
| 1 | GND-EARTH | Power |  |
| 2 | NC | Power |  |
| 3 | PWR-GND | Power |  |
| 4 | +VALIM | Power | 12-24V |

Table 4

### Power supply requirements

The following tables exhibite the current drawn from the input Power Supply connector when the operating voltage is set to 12 VDC, for both the board and the panel PC.

All the results consider the carrier board with a full-optional configuration.

These are theoretical values under an intensive use case, the actual current consumption depends on the hardware peripherals attached, the operating conditions and the running software.

| Board model | Max. Input current (at 12 V) [A] | Max. Power dissipation (at 12 V) [W] |
| --- | --- | --- |
| 3SB4000 (without display) | 2.5 | 30 |

Table 5 – 3SB4000 board with full-optional configuration, display excluded

| PC panel model | Max. Input current (at 12 V) [A] | Max. Power dissipation (at 12 V) [W] |
| --- | --- | --- |
| 3HI4001 | 3 | 46 |
| 3HI4005 | 3 | 46 |
| 3HI4011 | 3 | 46 |
| 3HI4021 | 4 | 58 |

Table 6 – 3HI40xx series with full-optional configuration

## SOM

The Agata HMi / Lea SBC board mount the Eva SoM based on the i.MX8M Plus Processor. For additional info about the SOM please refer to 3SM1009 Datasheet [4].

## Reset and Boot Configuration

### Power On Control

Agata HMI / Lea SBC boards auto start when power supply is applied.

### Reset Button (SW2)

SW2 button can be used to reset the board. When SW2 is pressed, SOM SYS_nRESET_IN reset signal is forced to 0 and board is kept in reset state until SW2 is released.

### Boot Mode Configuration Button (SW1)

SOM Boot mode configuration can be forced by the user using Boot Mode Configuration Button SW1.

| Condition | Selected Boot Mode |
| --- | --- |
| Power On Board with SW1 not pressed | Internal Boot Mode (Boot from eMMC) |
| Power On Board with SW1 pressed | Serial Downloader Mode (Recovery) |

Table 7

Please refer to document i.MX 8MP Reference Manual [1] or 3SM1009 Datasheet [4] for further information on boot configurations and parameters.

# I/O Interfaces

One of the main design goals for Agata HMI / Lea SBC boards was to maximize the number of interfaces and peripherals available for the user while keeping an overall small board footprint.

Due to i.MX 8MP CPU pin-muxing limitation, however, not all the interfaces described in the following section may be available simultaneously. Please check carefully if the required functionalities for your application are available.

The peripherals and connectors described in the following section may be optional depending on the selected ordering code.

## Debug USB Device FTDi (CN4) – micro USB Type B

Software is default configured to have System Debug Console on UART2 serial port. To simplify debugging and development there is an integrated FTDi UART to USB converter. The Debug USB FTDi connector uses a standard micro-USB Type B connector. After installation of FTDi virtual COM port drivers (VCP) on PC, connect a free PC USB port to CN4. A serial port will be enumerated, open with a terminal emulator program with 115200 8N1 settings to get the board debug output.

## 10/100/1000 Ethernet Connectors (CN17, CN16)

i.MX 8MP implements a triple-speed 10/100/1000-Mbit/s Ethernet MAC compliant with the IEEE802.3-2002 standard. The MAC layer provides compatibility with half- or full duplex 10/100-Mbit/s and full duplex gigabit Ethernet LANs. It also implements a hardware acceleration block to optimize the performance of network controllers providing TCP/IP, UDP, and ICMP protocol services. The acceleration block performs critical functions in hardware, which are typically implemented with large software overhead.

RTL8211F gigabit Ethernet PHY is integrated on board. PHY main features are the following:

- Compliant with IEEE802.3

- Support Auto-negotiation

- Automatic polarity detection and correction

- Adaptive equalization

- Cross-talk cancellation

A unique MAC address is pre-programmed to the board.

CN16 and CN17 are standard RJ45 10/100/1000 Ethernet connector with integrated magnetic and led indicators.

Green led blinks to indicate ethernet activity. The other led is used to indicate current ethernet speed: when it is green it indicates 1000 Mbps current speed, when it is orange it indicates 100Mbps current speed otherwise when it is off it indicates 10 Mbps current speed.

PoE (Power over Ethernet) is not supported.

## microSD Card connector CN15 - GCT MEM2052-00-195-00-A

If selected in the ordering code, a microSD connector is available on the board.

The microSD connector is connected to i.MX 8MP SDIO channel SD1.

Following table shows the microSD connector pinout:

| Pin Number | Pin name | Description | I/O Type | Operating voltage | i.MX 8MP Ball | On-board Termination |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | SD1_DAT2 | SD1 data 2 signal | Input/Output | 3.3V | V29 | 10k Pull-up – 22R series |
| 2 | SD1_DAT3 | SD1 data 3 signal | Input/Output | 3.3V | V28 | 10k Pull-up – 22R series |
| 3 | SD1_CMD | SD1 command signal | Input/Output | 3.3V | W29 | 10k Pull-up – 22R series |
| 4 | SD_PWR | SD 3.3V power supply output | Power | 3.3V |  |  |
| 5 | SD1_CLK | SD1 clock signal | Output | 3.3V | W28 | 10k Pull-up -22R series |
| 6 | GND | Ground |  |  |  |  |
| 7 | SD1_DAT0 | SD1 data 0 signal | Input/Output | 3.3V | Y29 | 10k Pull-up – 22R series |
| 8 | SD1_DAT1 | SD1 data 1 signal | Input/Output | 3.3V | Y28 | 10k Pull-up – 22R series |
| 9 | CD (SAI3_RXFS) | Card Detect signal | Input | 3.3V | AJ19 | 10k Pull-up |
| 10 | SHIELD | microSD connector shield. Connected to GND-EARTH |  |  |  |  |

Table 8

## RTC

A low power RTC is installed on board, part number is NXP PCF85263ATL.

RTC is connected to CPU via I2C2 bus on address 0x51 (7-bit). Alarm and interrupt functions are supported. Following table shows CPU signal used for interrupt / alarm function:

| Signal Name | Description | Operating voltage | i.MX 8MP Function |
| --- | --- | --- | --- |
| RTC_nINT | Interrupt / Alarm active low signal from RTC to CPU | 3.3V | Signal comes from on-board gpio expander. GPIO name is RTC_nINT_gpio. |

Table 9

Standby battery current (with oscillator running) is rated at 320nA typ (when Vbat = 3.3V and Tamb = 25°C).

In order to keep time and data information when power is removed, a standard CR1216/CR1220/CR1225 battery is required on connector CN7 socket.

### RTC Battery Socket (CN7)

CN7 Battery socket supports standard CR1216/CR1220/CR1225 3V Coin Cell Battery.

## USB

Agata HMI/ Lea SBC boards provide up to two USB 3.0 Super Speed Host (on standard A-type connector) ports and one 2.0 High Speed Host (on standard micro USB B-type connector) port:

- One USB 3.0 Host port (CN13 A-type Connector) is connected to a Super Speed HUB on the board;

- One USB 3.0 Host port (CN14 A-type Connector) is connected to i.MX 8MP USB OTG1 port;

- One USB 2.0 Host port (CN1 B-type Connector) is connected to a Super Speed HUB on the board;

- One USB 3.0 Host port is on M.2 Key B, connected to a Super Speed HUB on the board.

Figure 6

Each 3.0 USB controller can operate in Super-Speed operation (5.0 Gbps), High-Speed operation (480 Mbps), Full Speed operation (12 Mbps) and Low Speed operation (1.5 Mbps).

The 2.0 USB controller can operate in High-Speed operation (480 Mbps), Full Speed operation (12 Mbps) and Low Speed operation (1.5 Mbps).

For more information on i.MX 8MP USB interfaces refer to document i.MX 8MP Reference Manual [1].

## RS-232/RS-485/RS-422 UART Connector (CN18) – Molex 43045-0600

Agata HMi / Lea SBC boards provide one UART port configurable by SW to RS-232 or RS-485 / RS-422 mode using an integrated RS-232 / RS-485 / RS-422 multi-standard transceiver. UART port is connected to i.MX 8MP internal UART1 port.

i.MX 8MP UART includes the following features:

- High-speed TIA/EIA-232-F compatible, up to 5.0 Mbit/s

- 7 or 8 data bits for RS-232 characters, or 9-bit RS-485 format

- 1 or 2 stop bits

- Programmable parity (even, odd, and no parity)

- Hardware flow control support for request to send (RTS_B) and clear to send (CTS_B) signals

- RS-485 driver direction control via CTS_B signal

- Two independent, 32-entry FIFOs for transmit and receive

Following GPIOs are used to configure multi-standard transceiver for UART1:

| Function | GPIO name | Note |
| --- | --- | --- |
| RS-232 / RS-485 mode selection | Signal comes from on-board gpio expander. GPIO name is GPIO_MODE. | - RS-232 Mode: set GPIO_MODE to 0 - RS-485 Mode: set GPIO_MODE to 1 |
| RS-485 Duplex Mode | Signal comes from on-board gpio expander. GPIO name is GPIO_DUPLEX_SEL. | Valid only when mode is set to RS-485 - Full Duplex when GPIO_DUPLEX_SEL is set to 0; - Half Duplex when GPIO_DUPLEX_SEL is set to 1; |
| Slew rate selection (to minimize EMI) | Signal comes from on-board gpio expander. GPIO name is GPIO_SLEW. | - Slew limitation to 250 kbps if GPIO_SLEW io set to zero - Data Rate up to 1Mbps in RS-232 mode and up to 20Mbps in RS-485/422 mode when GPIO_SLEW is set to 1. |
| Shutdown Mode | Signal comes from on-board gpio expander. GPIO name is GPIO_SHDN. | - Low Power shutdown mode is on when GPIO_SHDN is 0. - Low Power shutdown mode is off when GPIO_SHDN is 1. |
| ECHO | Signal comes from on-board gpio expander. GPIO name is GPIO_ECHO. | If set to 1 enables echo for transmitted characters. Only valid for RS-485 mode. For all other modes leave it to 0. |

Table 10

Following table shows CN18 pinout:

| Pin Number | Pin name | Description | I/O Type | Standard |
| --- | --- | --- | --- | --- |
| 1 | GND-EARTH | Shield Ground |  |  |
| 2 | UART1_RS232-CTS_RS485-B | RS-232 Mode: CTS signal RS-485 Full Duplex Mode: B signal RS-485 Half Duplex Mode: Not used | Input | RS-232 RS-485 |
| 3 | UART1_RS232-RTS_RS485-Y | RS-232 Mode: RTS signal RS-485 Full Duplex Mode: Y signal RS-485 Half Duplex Mode: A/Y signal | Output / Input | RS-232 RS-485 |
| 4 | UART1_RS232-RX_RS485-A | RS-232 Mode: RX signal RS-485 Full Duplex Mode: A signal RS-485 Half Duplex Mode: Not used | Input | RS-232 RS-485 |
| 5 | UART1_RS232-TX_RS485-Z | RS-232 Mode: TX signal RS-485 Full Duplex Mode: Z signal RS-485 Half Duplex Mode: B/Z signal | Output / Input | RS-232 RS-485 |
| 6 | GND | Ground |  |  |

Table 11

When in RS-485 / RS-422 mode, 120R termination can be enabled through J3 and J4 jumpers:

- J4 enables 120R termination between UART1_RS232-RX_RS485-A and UART1_RS232-CTS_RS485-B signals

- J3 enables 120R termination between UART1_RS232-TX_RS485-Z and UART1_RS232-RTS_RS485-Y signals

## RS-485/RS-422 UART Connector (CN20) – Molex 43045-0600

3HI40xx / 3SB4000 provides an additional RS-485 / RS-422 port connected to internal IMX 8MP UART port UART4.

Following table shows CN20 pinout:

| Pin Number | Pin name | Description | I/O Type | Standard |
| --- | --- | --- | --- | --- |
| 1 | GND-EARTH | Shield Ground |  |  |
| 2 | RS485-B | RS-485 Full Duplex Mode: B signal | Input | RS-485 |
| 3 | RS485-Y | RS-485 Full Duplex Mode: Y signal | Output | RS-485 |
| 4 | RS485-A | RS-485 Full Duplex Mode: A signal | Input | RS-485 |
| 5 | RS485-Z | RS-485 Full Duplex Mode: Z signal | Output | RS-485 |
| 6 | GND | Ground |  |  |

Table 12

Jumper J1 enables 120R termination between RS485-A and RS485-B signals.

RS-485 driver direction is controlled via GPIO5_IO05.

## HDMI Display Connector (CN21) - Molex 208658-1003

Agata HMI / Lea SBC board provides one HDMI video output port, compatible with HDMI v2.0a spec. The video pixel rates are typically from 25 MHz up to 297MHz. HDMI can support several audio standards.

Standard HDMI connector follows the standard HDMI pinout. Display Edid is read thanks to a dedicated I2C / DDC bus included in the pinout.

## CAN-FD connector (CN19) – Molex 43045-0400

Agata HMI / Lea SBC board supports one CAN interface obtained from CAN1 interface of the IMX 8MP processor.

CAN interface supports CAN2.0 B and CAN FD Physical Layer Transceiver Requirements. It is optimized for CAN FD at 2, 5 and 8 Mbps Operation.

The following table collects all the pins on CAN-FD port:

| Pin Number | Pin name | Description / Function | I/O Type |
| --- | --- | --- | --- |
| 1 | GND-EARTH | Shield Ground | Power |
| 2 | GND | Ground | Power |
| 3 | CAN-H | Can Bus 1 H Signal | Input/Output |
| 4 | CAN-L | Can Bus 1 L signal | Input/Output |

Table 13

## I/O Expansion connector (CN3) – Molex 87831-2020

Agata HMi / Lea SBC boards provide an I/O Expansion connector to simplify connection to external board or peripheral.

The following table shows the details of the expansion connector used on the board:

| Model no | Pin count | Details |
| --- | --- | --- |
| 87831-2020 | 20 | Milli Grid 10x2 p2.0, Ms, DR |

Table 14

Some I/O Expansion connector signals have a fixed function, other signals have a default configuration but can be customized by the user. Additional details are provided in the next subsections.

### I/O Expansion connector in default configuration

#### Board Power Supply Output

5V and 3.3V output supply are available on expansion connector to power an external board. The maximum total current available on both 5V and 3.3V outputs is 0.2A.

For safe operation, it is recommended not to exceed 50mA from each single power supply pin of the Expansion connector.

| Pin Number | Pin name | Description / Function | I/O Type | Operating voltage |
| --- | --- | --- | --- | --- |
| 1 | +5V0_VIO | +5V0 output supply for external expansion board | Power | +5.0V |
| 2 |  |  |  |  |
| 3 | +3V3_VIO | +3V3 output supply for external expansion board | Power | +3.3V |
| 4 |  |  |  |  |

Table 15

#### Ground

The following table shows ground pin available on the I/O Expansion connector. GND is used on board as main ground for all peripherals. It is connected on board to PWR-GND through filters.

It is suggested to connect all GND pin to Expansion board GND to optimize GND return path and signal shielding.

| Pin Number | Pin name | Description / Function | I/O Type |
| --- | --- | --- | --- |
| 18 | GND | Ground | Power |
| 19 |  |  |  |
| 20 |  |  |  |

Table 16

#### GPIOs

The following Expansion connector signals are default configured as GPIOs signals:

| Pin Number | Pin name | Description / Function | I/O Type | Operating voltage | i.MX 8MP Ball | On-board Termination |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | GPIO1_IO15 | GPIO1_IO15 | I/O | +3.3V | B5 | 33R series |
| 13 | GPIO1_IO01 | GPIO1_IO01 | I/O | +3.3V | E8 | 33R series |
| 17 | GPIO-EXP-0 | I/O Expansion pin | I/O | +3.3V | n/a | 33R series |
| 18 | GPIO-EXP-1 | I/O Expansion pin | I/O | +3.3V | n/a | 33R series |

Table 17

#### ECSPI

Agata HMI / Lea SBC boards provide two ECSPI ports on the I/O Expansion connector in its default configuration: ECSPI2 and ECSPI1

##### ECSPI2

| Pin Number | Pin name | Description / Function | I/O Type | Operating voltage | i.MX 8MP Ball | On-board Termination |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | SPI2_MISO | ECSPI2 MISO signal | Input | +3.3V | AH20 | 33R series |
| 8 | SPI2_MOSI | ECSPI2 MOSI signal | Output | +3.3V | AJ21 | 33R series |
| 7 | SPI2_SCLK | ECSPI2 Serial Clock | Output | +3.3V | AH21 | 33R series |
| 6 | SPI2_SS0 | ECSPI2 Chip Select 0 | Output | +3.3V | AJ22 | 33R series |

Table 18

##### ECSPI1

| Pin Number | Pin name | Description / Function | I/O Type | Operating voltage | i.MX 8MP Ball | On-board Termination |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | SPI1_MISO | ECSPI1 MISO signal | Input | +3.3V | AD20 | 33R series |
| 12 | SPI1_MOSI | ECSPI1 MOSI signal | Output | +3.3V | AC20 | 33R series |
| 11 | SPI1_SCLK | ECSPI1 Serial Clock | Output | +3.3V | AF20 | 33R series |
| 10 | SPI1_SS0 | ECSPI1 Chip Select 0 | Output | +3.3V | AE20 | 33R series |

Table 19

## Audio Connector (CN8) – Molex MicroFit 43045-0400

CN7 connector provides a stereo headphone audio output and a microphone input, obtained from NAU88C22YG audio codec connected to i.MX 8MP SAI3 port. Bias is provided at the microphone input and it is controllable via software:

| Pin Number | Pin name | Description | I/O Type |
| --- | --- | --- | --- |
| 1 | AUDIO-OUT-L | Headphone Right audio output | Analog output |
| 2 | MIC-IN | Microphone Input | Analog input |
| 3 | AUDIO-OUT-R | Headphone Left audio output | Analog output |
| 4 | GND-A | Audio Analog Ground | Ground |

Table 20

## Buzzer (BZ1)

A small 8.5mm x 8.5mm magnetic buzzer is available on 3HI40xx / 3SB4000 boards. Buzzer resonant frequency is 4kHz and sound pressure level is up to 100dB. Part number is: Same Sky CMT-8504-100-SMT-TR.

The buzzer is driven by CPU PWM3 signal.

## M.2 Key E (CN11)

Agata HMI / Lea SBC boards provide a standard M.2 key E connector to allow the insertion of custom M.2

Wi-Fi/BT modules.

If Wi-Fi option is selected in the ordering informations, Wi-Fi/BT module U-Blox M2-MAYA-W161-00C is pre-assembled on board.

SDIO channel SD2 and UART3 are used for module connection.

## M.2 Key B (CN10)

Agata HMI / Lea SBC boards provide a standard M.2 key B connector to allow the insertion of custom M.2

LTE modules.

If LTE option is selected in the ordering informations LTE module Quectel EM05EFA-512-SGAS is pre-assembled on board.

USB Super Speed port from integrated USB HUB and audio channel SAI5 are used for module connection.

## Camera (CN2) – Hirose FH41-28S-0.5SH

Digital camera connector CN2 is designed to connect to a MIPI CSI-2 camera with up to 4 lanes.

CN2 connector provides 5V power supply for the camera and an I2C bus for camera control (connected to CPU I2C3 bus).

The following table shows camera connector pin assignment:

| Pin Number | Pin name | Description / Function | I/O Type | Operating voltage | On-board Termination |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | Ground |  |  |  |
| 2 | CSI1_D3_DP | MIPI CSI-2 Lane 3 | Diff. Input | MIPI CSI-2 |  |
| 3 | CSI1_D3_DN |  | Diff. Input |  |  |
| 4 | GND | Ground |  |  |  |
| 5 | CSI1_D2_DP | MIPI CSI-2 Lane 2 | Diff. Input | MIPI CSI-2 |  |
| 6 | CSI1_D2_DN |  | Diff. Input |  |  |
| 7 | GND | Ground |  |  |  |
| 8 | CSI1_CLK0_DP | MIPI CSI-2 Clock Lane | Diff. Input | MIPI CSI-2 |  |
| 9 | CSI1_CLK0_DN |  | Diff. Input |  |  |
| 10 | GND | Ground |  |  |  |
| 11 | CSI1_D1_DP | MIPI CSI-2 Lane 1 | Diff. Input | MIPI CSI-2 |  |
| 12 | CSI1_D1_DN |  | Diff. Input |  |  |
| 13 | GND | Ground |  |  |  |
| 14 | CSI1_D0_DP | MIPI CSI-2 Lane 0 | Diff. Input | MIPI CSI-2 |  |
| 15 | CSI1_D0_DN |  | Diff. Input |  |  |
| 16 | GND | Ground |  |  |  |
| 17 | NC | Not Connected |  |  |  |
| 18 | NC | Not Connected |  |  |  |
| 19 | GND | Ground |  |  |  |
| 20 | I2C3_SCL | I2C for Camera Control | Output | 1.8V | 4k7 Pullup |
| 21 | I2C3_SDA |  | Bidir | 1.8V | 4k7 Pullup |
| 22 | GND | Ground |  |  |  |
| 23 | NC | Not Connected |  |  |  |
| 24 | NC | Not Connected |  |  |  |
| 25 | +5V_CAMERA | Camera Supply Output | Power Output | 5.0V |  |
| 26 | +5V_CAMERA | Camera Supply Output | Power Output | 5.0V |  |
| 27 | +5V_CAMERA | Camera Supply Output | Power Output | 5.0V |  |
| 28 | GND | Ground |  |  |  |

Table 21

## nanoSIM Card Slot (CN23) – TE 2336582-1

This is a standard nanoSIM connector with the following pinout:

| Pin Number | Pin name | Description / Function | I/O Type | Operating voltage | On-board Termination |
| --- | --- | --- | --- | --- | --- |
| 1 | M2_SIM_PWR | SIM Power |  |  |  |
| 2 | M2_SIM_RST | SIM Reset | I |  | 22R0 series |
| 3 | M2_SIM_CLK | SIM Clock | I |  | 22R0 series |
| 4 | SHIELD | Shield |  |  |  |
| 5 | GND | Ground |  |  |  |
| 6 | NC | Not Connected |  |  |  |
| 7 | M2_SIM_DATA | SIM Data | I/O |  | 22R0 series |
| 8 | M2_SIM_DETECT | SIM Detect | O |  | 22R0 series |

Table 22

SIM Signals are routed to M.2 Key B connector for LTE module. If LTE module is not installed, SIM connector is not used.

# Ordering Information

## Lea SBC Ordering Codes

1 Configuration 2 include only the following sections: 2x Giga LAN Connectors, SIM connector, RTC, UART debug

connector, 2X USB connectors, HDMI connector.

## Agata HMI Ordering Codes

(*) Some ordering code combinations are not allowed, for more information on ordering codes please contact Elettronica GF sales department.

# Legal notes

Elettronica GF does not grant any license (express or implied) under any patent right, copyright, mask work right or other intellectual property right of Elettronica GF covering or relating to any combination, machine, or process in which its products or services might be or are used. Any provision or publication of any third party’s products or services does not constitute Elettronica GF’s approval, license, warranty or endorsement thereof.

Elettronica GF guarantees hardware product against defects in workmanship and material for a period of one (1) year from the date of shipment. Your sole remedy and Elettronica GF’s sole liability shall be for Elettronica GF, at its sole discretion, to either repair or replace the defective hardware product at no charge or to refund the purchase price. Shipment costs in both directions are the responsibility of the customer. This warranty is void if the hardware product has been altered or damaged by accident, misuse or abuse.

Elettronica GF will not liable for any indirect, special, incidental or consequential damages of any kind or nature, including, without limitation, business interruption costs, loss of profit or revenue, loss of data, promotional or manufacturing expenses, overhead, costs or expenses associated whit warranty or intellectual property infringement claims, injury to reputation or loss of customers.

The product, including associated software and documentations, is not designed, authorized or warranted to be suitable for use in medical, military, aircraft, space or life support equipment nor in applications where failure or malfunction of the product can reasonably be expected to result in a personal injury, death or severe property or environmental damage. (conjointly, "High Risk Applications").

You understand and agree that your use of Elettronica GF product as a component in High Risk Applications is entirely at your risk.

Customer’s recovery from Elettronica GF for any claim shall not exceed customer’s purchase price for the product giving rise to such claim irrespective of the nature of the claim, whether in contract, tort, warranty, or otherwise.

© Copyright 2024, Elettronica GF s.r.l. con socio unico - All Rights Reserved.

# Contact Information

Elettronica GF s.r.l. con socio unico

Via Cà Bianca, 10 – 48018 Faenza (Ra) – Italy

Tel. (+39) 0546 622245 (r.a.)

Fax (+39) 0546 622301

website: 		www.elettronicagf.it

technical support: 	technical.service@elettronicagf.it

sales: 			sales@elettronicagf.it

# Annex A - Safety and Environmental Requirements

## EMC

Lea SBC and Agata HMI boards have been designed according to the standards EMC (electromagnetic compatibility). Depending on the target system, additional anti-interference precautions may be necessary.

## ESD

ESD preventive measures are provided on Lea SBC and Agata HMI boards for all connectors.

Handle board and its parts with care. Be sure to be grounded to avoid electrostatic discharges which may cause irreversible damage to the device.

## Environmental operating conditions

- Operating temperature:

| Model | Commercial Grade1 | Industrial Grade1 |
| --- | --- | --- |
| 3SB4000 | 0 °C to +70 °C | -40 °C to +85 °C |
| 3HI40xx | 0 °C to +50 °C | -20 °C to +70 °C |

Table 23

- Relative air humidity (operation / storage): 	10 % to 90 % (not condensing)

________________________________________________________________________________

1 These temperatures are the ambient operating temperature ranges for the components used into Lea SBC / Agata HMI series, with exception of iMX8M Plus processor, which is specified on junction operating temperature range, 0-95°C for commercial version and -40-105°C for industrial version. Ambient operating temperature depends on the application and on the cooling measures applied

The 3SB4000 and 3HI40xx boards allowed ambient operating temperature depends on the application and on the cooling measures applied. Customer should consider specific thermal design for the final product based upon the specific environmental and operational conditions.

The thermal design should take into account at least the following criteria:

- Software use case

- Power consumption (results mainly from required calculating power and peripherals activated)

- Surrounding temperatures and existing airflow in case the system is mounted into a housing

- Thermal resistance of the heat dissipation paths within the system

## RoHS compliance

Lea SBC, Agata HMI boards are RoHS compliant (all the used components and soldering processes are RoHS compliant).

## PCB Specification

PCB Code: PGF0890

Base material:  FR4

UL flame class: UL94V-0

## Batteries

A Standard CR1216 / CR1220 / CR1225 3V Coin Cell Battery could be used on 3HI40xx / 3SB4000 boards.

# Annex B - Mechanical drawing

## Lea SBC board mechanical drawing

Figure 7: Lea SBC Layout – Top Side (left image) and Left Side (right image). All dimensions are expressed in mm.

## Agata HMI board mechanical drawing

### Agata HMIxxxxxP

Drawing available soon.

### Agata HMIxxxxxQ

Drawing available soon.

### Agata HMIxxxxxR

Drawing available soon.

### Agata HMIxxxxxP

Drawing available soon.

FRONT VIEW

Agata HMIxxxxxQ

SIDE VIEW

TOP VIEW

BOTTOM VIEW

REAR VIEW

Figure 8: 3HI4005xxxxxQ Layout. All dimensions are in mm.

### Agata HMI11xxxxxP

SIDE VIEW

FRONT VIEW

REAR VIEW

BOTTOM VIEW

Figure 9: 3HI4011xxxxxP Layout. All dimensions are in mm.

### Agata HMI21xxxxxP

REAR VIEW

Figure 10: 3HI4021xxxxxP Layout. All dimensions are in mm.

BOTTOM VIEW

SIDE VIEW

FRONT VIEW

# Annex C – Optical Characteristics

## Display 7.1” (Agata HMI1)

Specifications available soon.

## Display 10.1” (Agata HMI5xxxxxP / Agata HMI5xxxxxQ)

Test conditions:

- Panel lighting time: 30 minutes

- DVDD=3.3 V, IL=330 mA, Ta=25 °C

| Item | Symbol | Condition | Value |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min. | Typ. | Max. |  |
| Viewing angle (CR ≥ 10) | θL | φ=180° (9 o’clock) | 80 | 85 | 89 | degree |
|  | θR | φ=0° (3 o’clock) | 80 | 85 | 89 |  |
|  | θT | φ=90° (12 o’clock) | 80 | 85 | 89 |  |
|  | θB | φ=270° (6 o’clock) | 80 | 85 | 89 |  |
| Response time | TON | Normal θ=φ=0° | -- | 10 | 20 | ms |
|  | TOFF |  | -- | 15 | 30 | ms |
| Contrast ratio | CR |  | 800 | 1000 | -- | -- |
| Color chromaticity | Wx |  | 0.26 | 0.31 | 0.36 | -- |
|  | Wy |  | 0.28 | 0.33 | 0.38 |  |
| Luminance | L |  | 350 | 450 | -- | cd/m2 |
| Luminance uniformity | BUNI |  | 70 | 75 | -- | % |

Table 24:  Information taken from the display datasheet, under the specified test conditions.

## Display 10.1” (Agata HMI5xxxxxR)

Test conditions:

- Panel lighting time: 30 minutes

- VDD=3.3 ± 0.3 V, Ta=25 °C

| Item | Symbol |  | Condition | Value |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min. | Typ. | Max. |  |
| Viewing angle range | Hor. | θ3 | CR > 10 | -- | 80 | -- | degree |
|  |  | θ9 |  | -- | 80 | -- |  |
|  | Vert. | θ12 |  | -- | 80 | -- |  |
|  |  | θ6 |  | -- | 80 | -- |  |
| Color gamut (C light) | -- |  | -- | -- | 50 | -- | % |
| Luminance contrast ratio | CR |  | θ=0° | -- | 800 | -- | -- |
| Transmittance | T(%) |  |  | -- | 6.6 | -- | % |
| White chromaticity | xW |  |  | 0.235 | 0.287 | 0.337 | -- |
|  | yW |  |  | 0.263 | 0.313 | 0.363 |  |
| Reproduction of color (C light) | xR (red) |  |  | 0.538 | 0.588 | 0.638 |  |
|  | yR (red) |  |  | 0.298 | 0.348 | 0.398 |  |
|  | xG (green) |  |  | 0.281 | 0.331 | 0.381 |  |
|  | yG (green) |  |  | 0.533 | 0.583 | 0.633 |  |
|  | xB (blue) |  |  | 0.101 | 0.151 | 0.201 |  |
|  | yB (blue) |  |  | 0.075 | 0.125 | 0.175 |  |
| Response time (Rising + Falling) | TRT |  | Ta=25 °C θ=0° | -- | 8 | -- | ms |

Table 25: Information taken from the display datasheet, under the specified test conditions.

## Display 15.6” (Agata HMI11)

Test conditions:

- Panel lighting time: 20 minutes

| Item | Symbol |  |  | Condition | Value |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min. | Typ. | Max. |  |
| Contrast ratio | CR |  |  | θX=0°, θY=0° Viewing normal angle | 800 | 1000 | -- | -- |
| Response time | TR |  |  |  | -- | 14 | 16 | ms |
|  | TF |  |  |  | -- | 11 | 14 |  |
| Average luminance of white | LAVE |  |  |  | 255 | 300 | -- | cd/m2 |
| Color chromaticity | Red |  | Rx |  | Typ. – 0.03 | 0.658 | Typ. + 0.03 | -- |
|  |  |  | Ry |  |  | 0.328 |  |  |
|  | Green |  | Gx |  |  | 0.275 |  |  |
|  |  |  | Gy |  |  | 0.574 |  |  |
|  | Blue |  | Bx |  |  | 0.143 |  |  |
|  |  |  | By |  |  | 0.092 |  |  |
|  | White |  | Wx |  |  | 0.314 |  |  |
|  |  |  | Wy |  |  | 0.361 |  |  |
| Color gamut | C.G |  |  |  | 64 | 69 | -- | % |
| Viewing angle | Horizontal | θX+ |  | CR ≥ 10 | 80 | 89 | -- | degree |
|  |  | θX- |  |  | 80 | 89 | -- |  |
|  | Vertical | θY+ |  |  | 80 | 89 | -- |  |
|  |  | θY- |  |  | 80 | 89 | -- |  |
| White variation |  | δW5p |  | θX=0°, θY=0° | 80 | -- | -- | % |
|  |  | δW13p |  |  | 65 | -- | -- |  |

Table 26: Information taken from the display datasheet, under the specified test conditions.

## Display 21.5” (Agata HMI21)

Test conditions:

- Panel lighting time: 30 minutes

- VDD=5.0 V, Fv=60 Hz, Is=65 mA, Ta=25 °C

| Item | Symbol |  | Condition | Value |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min. | Typ. | Max. |  |
| Contrast ratio | CR |  | Center of screen | 2000 | 3000 | -- | -- |
| Horizontal viewing angle | Right | θR | CR=10 | 75 | 89 | -- | degree |
|  | Left | θL |  | 75 | 89 | -- |  |
| Vertical viewing angle | Up | φH |  | 75 | 89 | -- |  |
|  | Down | φL |  | 75 | 89 | -- |  |
| Horizontal viewing angle | Right | θR | CR=5 | 75 | 89 | -- |  |
|  | Left | θL |  | 75 | 89 | -- |  |
| Vertical viewing angle | Up | φH |  | 75 | 89 | -- |  |
|  | Down | φL |  | 75 | 89 | -- |  |
| Response time | TR |  | -- | -- | 13 | 28 | ms |
|  | TF |  |  | -- | 5 | 8 |  |
|  | TR+TF |  |  | -- | 18 | 36 |  |
| Color coordinates (CIE 1931) | Red x | Rx | -- | 0.622 | 0.652 | 0.682 | -- |
|  | Red y | Ry |  | 0.305 | 0.335 | 0.365 |  |
|  | Green x | Gx |  | 0.291 | 0.321 | 0.351 |  |
|  | Green y | Gy |  | 0.595 | 0.625 | 0.655 |  |
|  | Blue x | Bx |  | 0.123 | 0.153 | 0.183 |  |
|  | Blue y | By |  | 0.037 | 0.067 | 0.097 |  |
|  | White x | Wx |  | 0.285 | 0.305 | 0.335 |  |
|  | White y | Wy |  | 0.298 | 0.318 | 0.348 |  |
| Crosstalk | CT |  | -- | -- | -- | 2.0 | % |
| Flicker | FdB |  | Center of screen | -- | -- | -20 | dB |
| Luminance of white | YL |  | -- | 280 | 300 |  | cd/m2 |

Table 27: Information taken from the display datasheet, under the specified test conditions.