- IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

Product Identifier

Product Name: 		DryQuat

Product Code:		012-01

PCPA Registration No.:	35944

Relevant identified uses of the substance or mixture and uses advised against

Product Use:		Herbicide

Restrictions on use:		See product label for restrictions

Details of the supplier of the safety data sheet

Company:		Life Scientific (Canada) Inc.

525-8th Avenue S.W., 43rd Floor

Eighth Avenue Place East

Calgary, Alberta T2P 1G1

Telephone:		403-498-6623

Email:			info@lifescientific.com

Web:			www.lifescientific.com

Emergency contact information

In case of Emergency:	1-800-424-9300

- HAZARD IDENTIFICATION

DANGER

May be corrosive to metals. (H290)

Harmful if swallowed. (H302)

Causes serious eye irritation. (H319)

Toxic if inhaled. (H331)

May cause respiratory irritation. (H335)

Causes damage to organs (Eyes) through prolonged or repeated exposure. (H372)

HAZARD CLASSIFICATION

| Health Hazards | Category | Physical Hazards | Category |
| --- | --- | --- | --- |
| Acute Tox. (oral) | 4 | Corrosive to metals | 1 |
| Acute Tox. (inhalation) | 3 |  |  |
| Eye irritation | 2A | Environmental Hazards | Category |
| STOT-SE | 3 (Respiratory system) | None | - |
| STOT-RE | 1 (Eyes) |  |  |

HAZARDS NOT REQUIRING CLASSIFICATION

None

Precautionary Statements

Keep only in original packaging. (P234)

Do not breathe mist or vapours. (P260)

Wash skin thoroughly after handling. (P264)

Do not eat, drink or smoke when using this product. (P270)

Use only outdoors or in a well-ventilated area. (P271)

Wear eye protection/ face protection. (P280)

IF SWALLOWED: Call a POISON CENTER/ doctor if you feel unwell. Rinse mouth. (P301 + P312 + P330)

IF INHALED: Remove person to fresh air and keep comfortable for breathing. Call a POISON CENTER/ doctor. (P304 + P340 + P311)

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. (P305 + P351 + P338)

Get medical advice/ attention if you feel unwell. (P314)

If eye irritation persists: Get medical advice/ attention. (P337 + P313)

Absorb spillage to prevent material damage. (P390)

Store in a well-ventilated place. Keep container tightly closed. (P403 + P233)

Store locked up. (P405)

Dispose of contents/ container to an approved waste disposal plant. (P501)

- COMPOSITION / INFORMATION ON INGREDIENTS

| Chemical Name | CAS | Concentration (% w/w) |
| --- | --- | --- |
| Diquat dibromide | 85-00-7 | 31.93 |
| Alkylamine ethoxylate | 70955-14-5 | >= 1 - < 5 |

- FIRST AID MEASURES

Description of first aid measures

General information:	Have the product container, label or Safety Data Sheet with you when calling the emergency number, a Poison Control Centre or physician, or going for treatment.

Inhalation:	Move to fresh air. If breathing is irregular or stopped, administer artificial respiration. Keep patient warm and at rest. Call a physician or Poison Control Centre immediately.

Ingestion:	If swallowed, seek medical advice immediately and show this container or label. Do NOT induce vomiting.

Skin contact:	Take off all contaminated clothing immediately. Wash off immediately with plenty of water. If skin irritation persists, call a physician. Wash contaminated clothing before re-use.

Eye contact:	Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Remove contact lenses. Immediate medical attention is required.

Most important symptoms and effects, both acute and delayed

Symptoms:	Inflammation of the mouth, throat and oesophagus

Gastrointestinal discomfort

Diarrhoea

Harmful if swallowed.

Causes serious eye irritation.

Toxic if inhaled.

May cause respiratory irritation.

Causes damage to organs through prolonged or repeated exposure

Indication of any immediate medical attention and special treatment needed

Note to physicians: 	Administer either activated charcoal (100g for adults or 2g/kg body weight in children) or Fuller's Earth (15% solution; 1 litre for adults or 15ml/kg body weight in children). NOTE: The use of gastric lavage without administration of an adsorbent has not shown any clinical benefit.

Eye contact: - Severe damage may be caused by apparently trivial contact and healing may be delayed. Medical supervision should continue until complete healing has occurred.

- FIREFIGHTING MEASURES

Extinguishing media

Suitable:		Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide (Small fires)

Alcohol-resistant foam or Water spray (Large fires)

Unsuitable: 	Do not use a solid water stream as it may scatter and spread fire.

Special hazards arising from the substance or mixture

As the product contains combustible organic components, fire will produce dense black smoke containing hazardous products of combustion (see section 10). Exposure to decomposition products may be a hazard to health.

Hazardous combustion products: Carbon oxides, Nitrogen oxides (NOx), Bromine compounds

Advice for firefighters

Do not allow run-off from firefighting to enter drains or water courses. Cool closed containers exposed to fire with water spray. Wear full protective clothing and self-contained breathing apparatus.

- ACCIDENTAL RELEASE MEASURES

Personal precautions, protective equipment and emergency procedures

Refer to protective measures listed in sections 7 and 8.

Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not flush into surface water or sanitary sewer system. If the product contaminates rivers and lakes or drains inform respective authorities.

Methods and materials for containment and cleaning up

Contain spillage, and then collect with non-combustible absorbent material, (e.g. sand, earth, diatomaceous earth, vermiculite) and place in container for disposal according to local / national regulations (see section 13). Clean contaminated surface thoroughly. Clean with detergents. Avoid solvents. Retain and dispose of contaminated wash water.

- HANDLING AND STORAGE

Precautions for safe handling

Avoid contact with skin and eyes. When using do not eat, drink or smoke. For personal protection see section 8. Spray solutions should not be mixed, stored or applied in containers other than plastic, plastic-lined steel, stainless steel or fiberglass.

Conditions for safe storage, including any incompatibilities

No special storage conditions required. Keep containers tightly closed in a dry, cool and well-ventilated place. Keep out of the reach of children. Keep away from food, drink and animal feeding stuffs.

- EXPOSURE CONTROL/PERSONAL PROTECTION

Control parameters

| Component | Value type (Form of exposure) | Control parameters | Source |
| --- | --- | --- | --- |
| Diquat dibromide | TWA (Total) | 0.5 mg/m3 | CA AB OEL |
|  | TWA (Respirable) | 0.1 mg/m3 | CA AB OEL |
|  | TWA | 0.5 mg/m3 | CA ON OEL |
|  | TWA (Respirable fraction) | 0.1 mg/m3 | CA ON OEL |
|  | TWAEV (respirable dust) | 0.1 mg/m3 (the cation) | CA QC OEL |
|  | TWA (Inhalable particulate matter) | 0.5 mg/m3 (the cation) | ACGIH |
|  | TWA (Respirable particulate matter) | 0.1 mg/m3 (the cation) | ACGIH |

Exposure controls

Engineering measures	Containment and/or segregation is the most reliable technical protection measure if exposure cannot be eliminated. The extent of these protection measures depends on the actual risks in use. Maintain air concentrations below occupational exposure standards. Where necessary, seek additional occupational hygiene advice.

Respiratory protection:	When workers are facing concentrations above the exposure limit, they must use appropriate certified respirators. Suitable respiratory equipment: Respirator with a half face mask. The filter class for the respirator must be suitable for the maximum expected contaminant concentration (gas/vapour/aerosol/particulates) that may arise when handling the product. If this concentration is exceeded, self-contained breathing apparatus must be used.

Hand protection:	Wear protective gloves. The choice of an appropriate glove does not only depend on its material but also on other quality features and is different from one producer to the other. Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves. Also take into consideration the specific local conditions under which the product is used, such as the danger of cuts, abrasion, and the contact time. The breakthrough time depends amongst other things on the material, the thickness and the type of glove and therefore has to be measured for each case. Gloves should be discarded and replaced if there is any indication of degradation or chemical breakthrough.

Skin protection:	Choose body protection in relation to its type, to the concentration and amount of dangerous substances, and to the specific workplace. Remove and wash contaminated clothing before re-use. Wear as appropriate: Impervious clothing

Eye protection:	Tightly fitting safety goggles Always wear eye protection when the potential for inadvertent eye contact with the product cannot be excluded.

Protective measures:	The use of technical measures should always have priority over the use of personal protective equipment. When selecting personal protective equipment, seek appropriate professional advice.

- PHYSICAL AND CHEMICAL PROPERTIES

Information on basic physical and chemical properties

Physical state		Liquid.

Colour:			red brown to dark brown

Odour:			No data available

Melting point (°C):		No data available

Boiling point (°C):		No data available

Flammability: 		No data available

Lower and upper exp. Limit:	No data available

Flash Point:		Not applicable

Auto. Ignition temp (°C):	> 650 °C

Decomposition temp (°C):	No data available

pH (at 20 °C):		6-7

Viscosity: 		1.23 mPa(s)

Solubility:			No data available

Vapour pressure:		No data available

Density and/or relative den.:	1.17 g/cm³ (20 °C)

Relative vapour density:	No data available

Explosivity		Not explosive

Data based on a similar composition.

Other Information

None.

- STABILITY AND REACTIVITY

Reactivity

See section "Possibility of hazardous reactions".

Chemical Stability

Stable under normal conditions.

Possibility of hazardous reactions

Corrosive in contact with metals

Conditions to avoid

No decomposition if used as directed.

Incompatible material

Aluminium

Mild steel

Iron

Hazardous decomposition products

No hazardous decomposition products are known.

- TOXICOLOGICAL INFORMATION

The results are based on a similar composition.

Information on likely routes of exposure

Ingestion

Inhalation

Skin contact

Eye contact

Information on toxicological effects

Acute Toxicity

Product:

LD50 oral (rat):			1,049 mg/kg

LC50 inhlalation (rat)			0.64 mg/l/4h

Test atmosphere: dust/mist

Assessment: The substance/mixture is not toxic on inhalation as defined by dangerous goods regulations.

Remarks: Nose bleeding and soreness of the throat may result from spray mist o dust trapped on the nasal mucosa. Based on data from similar materials

LD50 dermal (rat):			> 2,000 mg/kg

Components:

diquat dibromide:

LD50 oral (rat):			399.75 mg/kg

Remarks: Lethal dose for man is approximately 4-6g of diquat (equivalent to approximately 60mg/kg). May cause nausea, vomiting, abdominal pain and diarrhoea within a few hours of swallowing. Ulceration of lips, mouth, throat and intestine may follow within 24-48 hours. Kidney failure and liver damage may occur; in severe cases circulatory collapse; coma or death/cardiac arrest.

LC50 inhalation (rat)			0.226 mg/l/4h

Test atmosphere: dust/mist

LD50 dermal (rat):			792 mg/kg

The substance or mixture has no acute dermal toxicity

Alkylamine ethoxylate:

Acute oral toxicity: 			The component/mixture is moderately toxic after single ingestion.

Skin corrosion/irritation

Based on available data, the classification criteria are not met.

Product:

Species				Rabbit

Result 				Mild skin irritation

Components:

Diquat dibromide

Species: 				Rabbit

Result: 				Irritating to skin.

Remarks:	May also cause discoloration, cracking and loss of nails. Normal growth follows without delay

Alkylamine ethoxylate:

Result: 				Irritating to skin.

Serious eye damage/eye irritation

Causes serious eye irritation.

Product:

Species: 				Rabbit

Result: 				Eye irritation

Components:

Diquat dibromide:

Species: 				Rabbit

Result: 				Eye irritation

Remarks: 	This material has a delayed eye irritation effect. May lead to ulceration of cornea and conjunctival epithelium giving rise to secondary infection. Although healing may be slow, the injury is superficial and with proper medical care recovery will be complete, even in severe cases.

Alkylamine ethoxylate:

Result: 				Risk of serious damage to eyes.

Respiratory or skin sensitisation

Skin sensitisation

Based on available data, the classification criteria are not met.

Respiratory sensitisation

Not classified due to lack of data.

Product:

Test Type: 			Buehler Test

Species: 				Guinea pig

Result: 				Does not cause skin sensitisation.

Components:

Diquat dibromide:

Species: 				Guinea pig

Result: 				May cause sensitisation by skin contact.

Germ cell mutagenicity

Not classified due to lack of data.

Components:

Diquat dibromide:

Germ cell mutagenicity - Assessment: 	Animal testing did not show any mutagenic effects.

Carcinogenicity

Not classified due to lack of data.

Components:

Diquat dibromide:

Carcinogenicity – Assessment		No evidence of carcinogenicity in animal studies.

Reproductive toxicity

Not classified due to lack of data.

Components:

Diquat dibromide:

Reproductive toxicity – Assessment	No toxicity to reproduction, No effects on or via lactation

STOT Specific target organ toxicity – single exposure

May cause respiratory irritation.

Components:

Diquat dibromide:

Assessment: 	The substance or mixture is classified as specific target organ toxicant, single exposure, category 3 with respiratory tract irritation

STOT Specific target organ toxicity – repeated exposure

Causes damage to organs (Eyes) through prolonged or repeated exposure.

Components:

Diquat dibromide:

Target Organs: 			Eyes

Assessment: 	The substance or mixture is classified as specific target organ toxicant, repeated exposure, category 1.

Remarks: 	Ocular effects (cataracts) have been reported following long term oral exposure of laboratory animals.

Aspiration hazard

Not classified due to lack of data.

- ECOLOGICAL INFORMATION

The results are based on a similar composition.

Product:

LC50 (Cyprinus carpio (Carp)): 	> 100 mg/l/96 h

EC50 (Daphnia magna (Water flea)): 	27.9 mg/l/ 48 h

ErC50 (Raphidocelis subcapitata 	0.153 mg/l/ 72 h

(freshwater green alga):

NOEC (Raphidocelis subcapitata 	0.022 mg/l/ 72 h

(freshwater green alga):

ErC50 		0.0152 mg/l/ 7 d

(Lemna gibba G3 (gibbous duckweed)):

NOEC 		0.00325 mg/l/ 7 d

(Lemna gibba G3 (gibbous duckweed)):	End point: Growth rate

Components:

Diquat dibromide:

LC50 (Oncorhynchus mykiss		10.46 mg/l/ 96 h

(rainbow trout)):

EC50 (Daphnia magna (Water flea)): 	2.49 mg/l/ 48 h

ErC50 (Navicula pelliculosa 		0.001148 mg/l/ 96 h

(Freshwater diatom)):

NOEC (Navicula pelliculosa		0.0005945 mg/l/ 96 h

(Freshwater diatom)):

NOEC (Pimephales promelas 		0.04726 mg/l/ 34 d

(fathead minnow)):

NOEC (Daphnia magna (Water flea)):	0.0504 mg/l/ 21 d

Persistence and Biodegradability

Components:

Diquat dibromide:

Stability in water: 			Degradation half-life: > 30 d

Remarks: Persistent in water.

Bioaccumulative potential

Components:

Diquat dibromide:

Bioaccumulation: 			Remarks: Low bioaccumulation potential.

Mobility in soil

Components:

Diquat dibromide:

Distribution among 			Remarks: immobile

environmental compartments

Stability in soil: 	Dissipation time: 11 - 41 yr

Percentage dissipation: 50 % (DT50)

Remarks: Persistent in soil.

Other adverse effects

Components:

Diquat dibromide:

Results of PBT and vPvB assessment: Substance is not persistent, bioaccumulative, and toxic (PBT). Substance is not very persistent and very bioaccumulative (vPvB).

- DISPOSAL CONSIDERATIONS

Waste treatment methods

Waste disposal procedures:	Refer to the product label for specific disposal/recycling information Do not contaminate ponds, waterways or ditches with chemical or used container. Do not dispose of waste into sewer. Where possible recycling is preferred to disposal or incineration. If recycling is not practicable, dispose of in compliance with local regulations.

Contaminated packaging:	Refer to the product label for specific disposal/recycling information

- TRANSPORT INFORMATION

Transport the product in accordance with the provisions of ADR for road, RID for rail, IMDG for the sea, and ICAO / IATA for air transport

UNRTDG

UN number: 			UN 1760

Proper shipping name: 		CORROSIVE LIQUID, N.O.S. (DIQUAT DIBROMIDE)

Class: 				8

Packing group: 			III

Labels: 				8

Environmentally hazardous: 		no

IATA-DGR

UN/ID No.: 			UN 1760

Proper shipping name: 		Corrosive liquid, n.o.s. (DIQUAT DIBROMIDE)

Class: 				8

Packing group: 			III

Labels: 				Corrosive

Packing instruction (cargo aircraft): 	856

Packing instruction (passenger aircraft): 852

IMDG-Code

UN number: 			UN 1760

Proper shipping name: 		CORROSIVE LIQUID, N.O.S. (DIQUAT DIBROMIDE)

Class: 				8

Packing group: 			III

Labels: 				8

EmS Code: 			F-A, S-B

Marine pollutant: 			yes

Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code

Not applicable for product as supplied.

National Regulations

TDG UN number: 			UN 3265

Proper shipping name: 	CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. (DIQUAT DIBROMIDE)

Class: 				8

Packing group: 			III

Labels : 8 			ERG

Code : 				153

Marine pollutant : 			yes (DIQUAT DIBROMIDE)

- REGULATORY INFORMATION

Safety, health and environmental regulation/legislation specific for the substance or mixture

PCP Registration No.		35944

PMRA Information: Read the label, authorized under the Pest Control Products Act, prior to using or handling the pest control product. This chemical is a pest control product regulated by Health Canada Pest Management Regulatory Agency and is subject to certain labelling requirements under the Pest Control Products Act. These requirements differ from the classification criteria and hazard information required for GHS-consistent safety data sheets.

There are Canada-specific environmental requirements for handling, use, and disposal of this pest control product that are indicated on the label.

- OTHER INFORMATION

Abbreviations and acronyms

ACGIH 		US. ACGIH Threshold Limit Values

ATE 		Acute toxicity estimate

CAS No. 		Chemical Abstracts Service number

CERCLA 		Comprehensive Environmental Response, Compensation, and Liability Act

EINECS 		European inventory of existing commercial substances

ELINCS 		European list of notified chemical substances

GHS		Globally harmonized system

IARC 		International Agency for Research on Cancer

IATA 		International Air Transport Association

ICAO		International Civil Aviation Organization

IMDG 		International Maritime Dangerous Goods

IMO 		International Maritime Organization

LC50		Lethal concentration to 50% of a test population

LD50		Lethal dose to 50% of a test population (median lethal dose)

NOEC		No observed effect concentration

N.O.S. 		Not otherwise specified

NTP 		US. National Toxicology Program (NTP) Report on Carcinogens

OECD 		Organization for Economic Co-operation and Development

PPE		Personal protective equipment

SDS		Safety data sheet

STOT RE		Specific target organ toxicity – repeated exposure

STOT SE		Specific target organ toxicity – single exposure

TDG 		Transportation of Dangerous Goods

TWA 		Time weighted average

UN 		United Nations

WHO 		World health organization

NFPA 704 (National Fire Protection Association):

Health - 0 Flammability - 0 Instability - 0 Others - none

HMIS (Hazardous Materials Identification System, based on the Third Edition Ratings Guide)

Health - 0 Flammability - 0 Physical Hazard - 0 PPE – 0

0 = minimal hazard, 1 = slight hazard, 2 = moderate hazard, 3 = severe hazard, 4 = extreme hazard

The information presented in this document is accurate to the best of our knowledge at the date of its publication. However, the information given is designed only as a guide for the methods of handling, storage, use, transportation and disposal of the product and is not considered a warranty or quality specification. Life Scientific Inc. cannot be held responsible for any loss or damage resulting from the handling, storage, use or disposal of the product. The information contained in this document relates only to this specific product.

DryQuat ® - trademark pending, Life Scientific Inc.

First Issuance: 		21.04.2026

Current Issuance: 		21/04/2026